| Technical Data Sheet Acetone Wipes |  |  |
| --- | --- | --- |

| Product Description |  | Typical LiquidProduct Data & Physical Properties |  |
| --- | --- | --- | --- |
| Acetone presaturated wipes are available in both dispensers and also in refill pouches. A highly efficient and versatile cleaner used in surface coatings, adhesives, dewaxing and degreasing. Combining an ultra-low lint base substrate with acetone to form a presaturated wipe means a large reduction on VOC emission and significant improvement in safety when compared to the traditional bottle and cloth application method. The product is supplied in a full UN dispensing container with a sealed pouch containing the product within. Available in a super heavyweight ultra-low lint fabric and also our standard weight ultra-low lint fabric which facilitates the shipping of the product in Limited Quantity. Country of origin : UK |  | Colour | Clear, colourless |
|  |  | Purity | >99.7% |
|  |  | Relative density @ 20⁰C | <1 |
|  |  | Boiling point | 56⁰C |
|  |  | Flash point | <18⁰C |
|  |  | Water miscibility | Complete |
|  |  | Specific gravity | 0.78 |
|  |  | Shelf life | 2 years |
|  |  | Melting point | -95⁰C |
|  |  | Auto ignition | 540⁰C |
|  |  | Base Fabric Data (Super Heavyweight) |  |
|  |  | Colour | White |
|  |  | Type | Spunlace |
|  |  | Composition | Polyester 48% Woodpulp 52% |
|  |  | Base weight | 68gsm |
|  |  | Absorbency | 490% |
|  |  | Tensile Strength | MD 167N |
|  |  |  | CD 75N |
|  |  | Thickness | 0.39mm |
|  |  | Standard Ultra-Low Lint (LQ shippable) |  |
|  |  | Colour | White |
|  |  | Type | Sontara® Style 9951 |
|  |  | Composition | Polyester 48% Woodpulp 52% |
|  |  | Base weight | 51gsm |
|  |  | Absorbency | 587% |
|  |  | Tensile strength | MD 124N |
|  |  |  | CD 53N |
|  |  | Thickness | 0.35mm |

| Part Number | Sheet size | Sheet Qty | Dispenser | Qty/Carton | Carton/pallet |
| --- | --- | --- | --- | --- | --- |
| CRC002 | 230x250 | 150 | UN Metal Pail | 6 | 48 |
| CRC004 | 230x250 | 130 | UN Metal Pail | 6 | 48 |

| Usage Instructions |  |  |  |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- |
| For professional use only Always refer to the MSDS |  |  |  |  | This information is believed to be accurate. It is intended for professional end users having the skills to evaluate and use the data correctly. Chemwipes does not guarantee the accuracy of the data and assumes no liability in connection with damages incurred whilst using it. |  |

|  |  |  |  | Manufactured By: |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Adamtech T/A Speciality Wipes 7B Whitebridge Industrial Estate Stone, Staffordshire, ST15 8LQ Tel : 00441785 286660 |  |