| Technical Data Sheet Eurotwill CW55 IPA/DI Wipes Revision: Orig Date: February 2017 |  |
| --- | --- |

| Product Description |  | Typical Product Data & Physical Properties |  |  |
| --- | --- | --- | --- | --- |
| IPA presaturated wipes may be used in a wide variety of applications for both industrial and laboratory use. The IPA/DI is 10 micron filtered and the base fabric is pharmaceutical grade. The wipes are conveniently packaged in a reseal pouch to give an extended shelf life of up to 2 years. Country of origin : UK |  | Colour |  | Clear, colourless |
|  |  | Purity |  | 99.8% |
|  |  | Relative density @ 20⁰C |  | 0.787 |
|  |  | Boiling point |  | 82⁰-83⁰C |
|  |  | Water content |  | 15% |
|  |  | Flash point |  | 12⁰C |
|  |  | Water miscibility |  | Complete |
|  |  | TLV |  | 400ppm |
|  |  | Specific gravity |  | 0.78 |
|  |  | Evaporation rate |  | <1 |
|  |  | Shelflife |  | 2 years |
|  |  | Melting point |  | -90⁰C |
|  |  | Auto ignition |  | 399⁰C |
|  |  | Base Fabric Data |  |  |
|  |  | Colour |  | White |
|  |  | Type |  | Duralace |
|  |  | Composition |  | 100% Viscose Rayon |
|  |  | Base weight |  | 55gsm |
|  |  | Tensile Strength |  |  |
|  |  |  | MD | 375N/12 plies |
|  |  |  | CD | 300N/12 plies |
|  |  | Thickness | 2.85 | Mm/4 plies |
|  |  | Absorption | 1000 | % |
|  |  | Sinktime | <2 | sec |

| Availability : | 25 sheet pouch |  | Compatibility |  |
| --- | --- | --- | --- | --- |
|  | 40 sheet vac pouch |  | IPA presaturated wipes are compatible with most materials although full compatibility must be tested in an inconspicuous area prior to use. |  |
| Sheet size | 43cmx31cm |  |  |  |

| Usage Instructions |  |  |  |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- |
| For professional use only Always refer to the MSDS |  |  |  |  | This information is believed to be accurate. It is intended for professional end users having the skills to evaluate and use the data correctly. Chemwipes does not guarantee the accuracy of the data and assumes no liability in connection with damages incurred whilst using it. |  |
| Environmental Impact |  |  |  |  |  |  |
| C FC | 0.0% | VOC | 85% |  |  |  |
| HCFC | 0.0% | HFC | 0.0% |  |  |  |
| CL Solv. | 0.0% | ODP | 0.0% |  |  |  |

| Pack Size | 1 | Pouch of 25 sheets |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Carton of 15 | : | Shipping weight | 14Kg |
|  |  | Euro Pallet of 24 cartons | : | Shipping weight | 350kg |
|  | 2 | Vac pouch of 50 sheet |  |  |  |
|  |  | Carton of 8 pouches |  |  |  |
|  |  | Pallet of 24 cartons |  |  |  |